/*

* SKETCH 3 – LoRa Receiver | Farm Gateway

* Receives LoRa packets from Transmitter.

* "S:..." sensor packets → LCD + ThingSpeak

* "I:..." image chunks  → reassemble → Cloudinary upload → Email

* ⚠ Requires WiFi. LoRa frequency MUST match Transmitter (433 MHz).

*/

#include <SPI.h>

#include <LoRa.h>

#include <WiFi.h>

#include <HTTPClient.h>

#include <WiFiClientSecure.h>

#include <ArduinoJson.h>

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

#include <ESP_Mail_Client.h>

// ─── LCD ──────────────────────────────────────────────────────────────────

#define LCD_ADDRESS 0x27

#define LCD_COLUMNS 16

#define LCD_ROWS    2

#define I2C_SDA     21

#define I2C_SCL     22

LiquidCrystal_I2C lcd(LCD_ADDRESS, LCD_COLUMNS, LCD_ROWS);

// ─── WiFi ─────────────────────────────────────────────────────────────────

const char *ssid     = "Algoredeem";

const char *password = "water100";

// ─── ThingSpeak ───────────────────────────────────────────────────────────

const char *thingspeakServer = "api.thingspeak.com";

String      apiKey           = "9ID8G1JSGUK17K4V";

// ─── Cloudinary ───────────────────────────────────────────────────────────

#define CLOUDINARY_CLOUD   "drcdr4kyx"

#define CLOUDINARY_PRESET  "take-pic"

#define CLOUDINARY_URL     "https://api.cloudinary.com/v1_1/" CLOUDINARY_CLOUD "/image/upload"

// ─── Email ────────────────────────────────────────────────────────────────

#define SMTP_HOST       "smtp.gmail.com"

#define SMTP_PORT       465

#define AUTHOR_EMAIL    "ld604068@gmail.com"

#define AUTHOR_PASSWORD "vpkmmnzfcyoeclfu"

#define RECIP_EMAIL     "msiateequ@yahoo.com"

SMTPSession      smtp;

ESP_Mail_Session mailSession;

SMTP_Message     mailMsg;

// ─── LoRa pins ────────────────────────────────────────────────────────────

#define LORA_MOSI 13

#define LORA_MISO 12

#define LORA_SCK  14

#define LORA_SS   15

#define LORA_RST  27

#define LORA_DIO0 26

// ─── LoRa settings ────────────────────────────────────────────────────────

#define LORA_FREQUENCY        433E6   // ⚠ Must match Transmitter!

#define LORA_SYNC_WORD        0xF3

#define LORA_SPREADING_FACTOR 12

#define LORA_BANDWIDTH        125E3

// ─── LCD custom characters ────────────────────────────────────────────────

byte temp_icon[8]     = {0b00100,0b01010,0b01010,0b01010,0b10001,0b10001,0b01110,0b00000};

byte humidity_icon[8] = {0b00100,0b00100,0b01010,0b01010,0b10001,0b10001,0b10001,0b01110};

byte plant_icon[8]    = {0b00100,0b01110,0b10101,0b00100,0b01110,0b11111,0b01110,0b00100};

#define CHAR_TEMP  0

#define CHAR_HUMID 1

#define CHAR_PLANT 2

// ─── Sensor data ──────────────────────────────────────────────────────────

struct ReceivedData {

int   packetNumber = 0;

float ph = 0, moisture = 0;

float temp_ds18b20 = 0, temp_bmp = 0;

float pressure = 0, temp_dht = 0, humidity = 0;

unsigned long timestamp = 0;

} rxData;

// ─── Image buffer ─────────────────────────────────────────────────────────

#define MAX_IMAGE_B64_SIZE 20480

static char     imgBuf[MAX_IMAGE_B64_SIZE];

static size_t   imgBufLen      = 0;

static uint16_t imgTotalChunks = 0;

static uint16_t imgChunksRxd   = 0;

static bool     imgComplete    = false;

// ─── Stats & timing ───────────────────────────────────────────────────────

int packetsReceived = 0, packetsSentTS = 0, failedTS = 0, imagesUploaded = 0;

int           displayMode       = 0;

unsigned long lastDisplaySwitch = 0;

unsigned long lastLCDUpdate     = 0;

unsigned long lastThingSpeak    = 0;

unsigned long lastWiFiCheck     = 0;

const unsigned long DISPLAY_SWITCH_MS = 3000;

const unsigned long LCD_UPDATE_MS     = 500;

const unsigned long TS_MIN_INTERVAL   = 15000;

const unsigned long WIFI_CHECK_MS     = 30000;

// ─── LCD ──────────────────────────────────────────────────────────────────

void initLCD() {

Wire.begin(I2C_SDA, I2C_SCL);

lcd.init();

lcd.backlight();

lcd.clear();

lcd.createChar(CHAR_TEMP,  temp_icon);

lcd.createChar(CHAR_HUMID, humidity_icon);

lcd.createChar(CHAR_PLANT, plant_icon);

}

void updateLCD() {

if (packetsReceived == 0) {

lcd.setCursor(0, 0); lcd.print("Waiting for data");

lcd.setCursor(0, 1); lcd.print("Pkts: 0         ");

return;

}

switch (displayMode) {

case 0:

lcd.setCursor(0, 0); lcd.write(CHAR_TEMP);

lcd.print(" T:"); lcd.print(rxData.temp_ds18b20, 1); lcd.print("C   ");

lcd.setCursor(0, 1); lcd.write(CHAR_HUMID);

lcd.print(" H:"); lcd.print(rxData.humidity, 1);

lcd.print("% P:"); lcd.print(rxData.packetNumber);

break;

case 1:

lcd.setCursor(0, 0); lcd.write(CHAR_PLANT);

lcd.print(" Mst:"); lcd.print(rxData.moisture, 1); lcd.print("%  ");

lcd.setCursor(0, 1);

lcd.print("pH:"); lcd.print(rxData.ph, 2);

lcd.print(" Pkt:"); lcd.print(rxData.packetNumber);

break;

case 2:

lcd.setCursor(0, 0); lcd.write(CHAR_TEMP);

lcd.print(" BMP:"); lcd.print(rxData.temp_bmp, 1); lcd.print("C ");

lcd.setCursor(0, 1);

lcd.print("Pr:"); lcd.print(rxData.pressure, 0);

lcd.print("hPa P:"); lcd.print(rxData.packetNumber);

break;

}

}

// ─── WiFi ─────────────────────────────────────────────────────────────────

bool connectWiFi() {

WiFi.mode(WIFI_STA);

WiFi.begin(ssid, password);

int tries = 0;

while (WiFi.status() != WL_CONNECTED && tries++ < 20) { delay(500); Serial.print("."); }

Serial.println();

if (WiFi.status() == WL_CONNECTED) {

Serial.print("WiFi OK: "); Serial.println(WiFi.localIP());

return true;

}

Serial.println("WiFi failed – offline mode");

return false;

}

void checkWiFi() {

if (WiFi.status() != WL_CONNECTED) connectWiFi();

}

// ─── LoRa init ────────────────────────────────────────────────────────────

bool initLoRa() {

SPI.begin(LORA_SCK, LORA_MISO, LORA_MOSI, LORA_SS);

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(LORA_FREQUENCY)) return false;

LoRa.setSyncWord(LORA_SYNC_WORD);

LoRa.setSpreadingFactor(LORA_SPREADING_FACTOR);

LoRa.setSignalBandwidth(LORA_BANDWIDTH);

return true;

}

// ─── Parse sensor packet (after "S:" prefix) ─────────────────────────────

bool parseSensor(const String &data) {

float v[8]; int idx = 0, start = 0;

int end = data.indexOf(',');

while (end != -1 && idx < 8) {

v[idx++] = data.substring(start, end).toFloat();

start = end + 1; end = data.indexOf(',', start);

}

if (start < (int)data.length() && idx < 8)

v[idx++] = data.substring(start).toFloat();

if (idx < 8) return false;

rxData.packetNumber = (int)v[0];

rxData.ph = v[1]; rxData.moisture = v[2];

rxData.temp_ds18b20 = v[3]; rxData.temp_bmp = v[4];

rxData.pressure = v[5]; rxData.temp_dht = v[6]; rxData.humidity = v[7];

rxData.timestamp = millis();

return true;

}

// ─── Parse image chunk (after "I:" prefix) ───────────────────────────────

// Format: "<chunkIdx>:<totalChunks>:<base64data>"

void parseImageChunk(const String &payload) {

int sep1 = payload.indexOf(':');

int sep2 = payload.indexOf(':', sep1 + 1);

if (sep1 < 0 || sep2 < 0) return;

uint16_t chunkIdx    = payload.substring(0, sep1).toInt();

uint16_t totalChunks = payload.substring(sep1 + 1, sep2).toInt();

String   b64data     = payload.substring(sep2 + 1);

if (chunkIdx == 0) {

imgBufLen = imgChunksRxd = 0;

imgTotalChunks = totalChunks;

imgComplete = false;

Serial.printf("[IMG] New image: %u chunks\n", totalChunks);

}

if (imgBufLen + b64data.length() < MAX_IMAGE_B64_SIZE) {

memcpy(imgBuf + imgBufLen, b64data.c_str(), b64data.length());

imgBufLen += b64data.length();

imgChunksRxd++;

Serial.printf("[IMG] Chunk %u/%u (%u bytes)\n", chunkIdx + 1, totalChunks, imgBufLen);

} else {

Serial.println("[IMG] Buffer overflow – chunk dropped");

}

if (imgChunksRxd >= totalChunks) {

imgBuf[imgBufLen] = '\0';

imgComplete = true;

Serial.printf("[IMG] Complete! %u bytes\n", imgBufLen);

uploadToCloudinary();

}

}

// ─── Cloudinary upload (base64 data URI) ─────────────────────────────────

void uploadToCloudinary() {

if (!imgComplete || imgBufLen == 0) return;

if (WiFi.status() != WL_CONNECTED) {

Serial.println("No WiFi – cannot upload"); imgComplete = false; return;

}

if (ESP.getFreeHeap() < imgBufLen + 8192) {

Serial.println("Not enough heap"); imgComplete = false; return;

}

WiFiClientSecure client;

client.setInsecure();

HTTPClient https;

https.setTimeout(30000);

if (!https.begin(client, CLOUDINARY_URL)) {

Serial.println("HTTPS begin failed"); imgComplete = false; return;

}

https.addHeader("Content-Type", "application/x-www-form-urlencoded");

String body = "upload_preset=" + String(CLOUDINARY_PRESET)

+ "&file=data:image/jpeg;base64," + String(imgBuf);

int    httpCode = https.POST(body);

String response = https.getString();

https.end();

imgComplete = false;

Serial.printf("[CLD] HTTP %d\n", httpCode);

if (httpCode == 200) {

DynamicJsonDocument doc(4096);

if (deserializeJson(doc, response) == DeserializationError::Ok) {

String url = doc["secure_url"].as<String>();

Serial.print("[CLD] URL: "); Serial.println(url);

imagesUploaded++;

sendEmail(url);

}

} else {

Serial.println("[CLD] Upload failed: " + response.substring(0, 200));

}

}

// ─── Email ────────────────────────────────────────────────────────────────

void sendEmail(const String &imageURL) {

mailSession.server.host_name = SMTP_HOST;

mailSession.server.port      = SMTP_PORT;

mailSession.login.email      = AUTHOR_EMAIL;

mailSession.login.password   = AUTHOR_PASSWORD;

mailMsg.clear();

mailMsg.sender.name  = "Farm Monitor Gateway";

mailMsg.sender.email = AUTHOR_EMAIL;

mailMsg.subject      = "Farm Snapshot";

mailMsg.addRecipient("Owner", RECIP_EMAIL);

String html =

"<div style='font-family:sans-serif;text-align:center;padding:20px'>"

"<h2>Farm Snapshot</h2>"

"<p><a href='" + imageURL + "' style='font-size:18px'>View Image</a></p>"

"<table style='margin:auto;border-collapse:collapse'>"

"<tr><td style='padding:4px 12px'>pH</td><td><b>"         + String(rxData.ph, 2)            + "</b></td></tr>"

"<tr><td>Moisture</td><td><b>"                            + String(rxData.moisture, 1)       + " %</b></td></tr>"

"<tr><td>Temperature</td><td><b>"                         + String(rxData.temp_ds18b20, 1)   + " C</b></td></tr>"

"<tr><td>Humidity</td><td><b>"                            + String(rxData.humidity, 1)       + " %</b></td></tr>"

"<tr><td>Pressure</td><td><b>"                            + String(rxData.pressure, 0)       + " hPa</b></td></tr>"

"</table></div>";

mailMsg.html.content = html.c_str();

mailMsg.html.charSet = "utf-8";

if (!smtp.connect(&mailSession)) {

Serial.println("[EMAIL] SMTP connect failed: " + smtp.errorReason()); return;

}

if (MailClient.sendMail(&smtp, &mailMsg)) Serial.println("[EMAIL] Sent");

else Serial.println("[EMAIL] Failed: " + smtp.errorReason());

}

// ─── ThingSpeak ───────────────────────────────────────────────────────────

void sendThingSpeak() {

unsigned long now = millis();

if (now - lastThingSpeak < TS_MIN_INTERVAL) return;

if (WiFi.status() != WL_CONNECTED) return;

HTTPClient http;

String url = "http://" + String(thingspeakServer)

+ "/update?api_key=" + apiKey

+ "&field1=" + String(rxData.ph, 2)

+ "&field2=" + String(rxData.moisture, 1)

+ "&field3=" + String(rxData.temp_ds18b20, 2)

+ "&field4=" + String(rxData.temp_bmp, 2)

+ "&field5=" + String(rxData.pressure, 2)

+ "&field6=" + String(rxData.temp_dht, 2)

+ "&field7=" + String(rxData.humidity, 2);

http.begin(url);

int code = http.GET();

if (code == 200) { packetsSentTS++; Serial.println("[TS] OK"); }

else             { failedTS++;      Serial.printf("[TS] Failed HTTP %d\n", code); }

http.end();

lastThingSpeak = now;

}

// ─── LoRa packet handler ──────────────────────────────────────────────────

void checkLoRa() {

int pktSize = LoRa.parsePacket();

if (!pktSize) return;

String raw = "";

while (LoRa.available()) raw += (char)LoRa.read();

Serial.printf("\n[RX] %d bytes: %s\n", pktSize, raw.substring(0, 60).c_str());

if (raw.startsWith("S:")) {

packetsReceived++;

if (parseSensor(raw.substring(2))) {

sendThingSpeak();

updateLCD();

}

} else if (raw.startsWith("I:")) {

parseImageChunk(raw.substring(2));

}

}

// ─── Setup / Loop ─────────────────────────────────────────────────────────

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("\nLoRa Receiver | Farm Gateway");

initLCD();

lcd.setCursor(0, 0); lcd.print("Farm Gateway");

lcd.setCursor(0, 1); lcd.print("Booting...");

delay(1500);

lcd.clear(); lcd.setCursor(0, 0); lcd.print("WiFi...");

connectWiFi();

lcd.setCursor(0, 1); lcd.print(WiFi.status() == WL_CONNECTED ? "WiFi: OK  " : "WiFi: FAIL");

delay(1000);

lcd.clear(); lcd.setCursor(0, 0); lcd.print("LoRa 433MHz...");

if (!initLoRa()) {

lcd.setCursor(0, 1); lcd.print("LoRa: FAIL!");

while (1) delay(1000);

}

lcd.setCursor(0, 1); lcd.print("LoRa: OK  ");

delay(1000);

lcd.clear();

lcd.setCursor(0, 0); lcd.print("Gateway Ready!");

lcd.setCursor(0, 1); lcd.print("SF12  433MHz");

delay(2000);

lcd.clear();

Serial.println("Gateway ready – listening\n");

}

void loop() {

unsigned long now = millis();

checkLoRa();

if (now - lastLCDUpdate >= LCD_UPDATE_MS) {

updateLCD(); lastLCDUpdate = now;

}

if (packetsReceived > 0 && now - lastDisplaySwitch >= DISPLAY_SWITCH_MS) {

if (++displayMode > 2) displayMode = 0;

lastDisplaySwitch = now;

updateLCD();

}

if (now - lastWiFiCheck >= WIFI_CHECK_MS) {

checkWiFi(); lastWiFiCheck = now;

}

delay(10);

}

/*

* SKETCH 2 – LoRa Transmitter | Sensor Node + ESP-NOW Relay

* Reads sensors every 2s, transmits via LoRa every 5s.

* Receives image base64 chunks from CAM via ESP-NOW and relays them over LoRa.

* Sensor packets : "S:<num>,<ph>,<moisture>,<tDS>,<tBMP>,<press>,<tDHT>,<humid>"

* Image packets  : "I:<chunkIdx>:<totalChunks>:<base64data>"

*/

#include <SPI.h>

#include <LoRa.h>

#include <Wire.h>

#include <Adafruit_BMP085.h>

#include <DHT.h>

#include <OneWire.h>

#include <DallasTemperature.h>

#include <WiFi.h>

#include <esp_now.h>

// ─── Sensor pins ──────────────────────────────────────────────────────────

#define PH_PIN       34

#define MOISTURE_PIN 35

#define DS18B20_PIN   4

#define DHT_PIN      25

#define DHT_TYPE     DHT11

#define I2C_SDA      21

#define I2C_SCL      22

// ─── LoRa pins (RA-02) ───────────────────────────────────────────────────

#define LORA_MOSI 13

#define LORA_MISO 12

#define LORA_SCK  14

#define LORA_SS   15

#define LORA_RST  27

#define LORA_DIO0 26

// ─── LoRa settings ────────────────────────────────────────────────────────

#define LORA_FREQUENCY        433E6   // ⚠ Must match Receiver!

#define LORA_SYNC_WORD        0xF3

#define LORA_TX_POWER         17

#define LORA_SPREADING_FACTOR 12

#define LORA_BANDWIDTH        125E3

#define LORA_CODING_RATE      5

// ─── Moisture calibration ────────────────────────────────────────────────

const int MOISTURE_DRY = 4095;

const int MOISTURE_WET = 1500;

// ─── Random pH range ─────────────────────────────────────────────────────

#define PH_MIN 6.5f

#define PH_MAX 7.5f

// ─── Image buffer (ESP-NOW → LoRa relay) ─────────────────────────────────

#define MAX_IMAGE_B64_SIZE 20480

#define LORA_IMG_CHUNK_BYTES  220

#define LORA_CHUNKS_PER_CYCLE   2

static char     imgBuf[MAX_IMAGE_B64_SIZE];

static size_t   imgBufLen        = 0;

static uint16_t imgTotalChunks   = 0;

static uint16_t imgChunksRxd     = 0;

static bool     imageReady       = false;

static uint16_t imgLoRaNextChunk = 0;

static bool     imgLoRaSending   = false;

// ─── ESP-NOW packet (must match Sketch 1) ────────────────────────────────

#define ESPNOW_CHUNK_BYTES 200

typedef enum : uint8_t {

PKT_IMAGE_CHUNK = 1,

PKT_IMAGE_END   = 2

} PktType;

struct __attribute__((packed)) EspNowPacket {

PktType  type;

uint16_t chunkIndex;

uint16_t totalChunks;

uint16_t dataLen;

char     data[ESPNOW_CHUNK_BYTES + 1];

};

// ─── Sensors ──────────────────────────────────────────────────────────────

DHT               dht(DHT_PIN, DHT_TYPE);

Adafruit_BMP085   bmp;

OneWire           oneWire(DS18B20_PIN);

DallasTemperature ds18b20(&oneWire);

bool bmpOK = false, dhtOK = false, dsOK = false;

struct SensorData {

float ph = 7.0f, moisture = 50.0f;

float temp_ds18b20 = 25.0f, temp_bmp = 25.0f;

float pressure = 1013.25f, temp_dht = 25.0f, humidity = 50.0f;

} sensors;

int           transmissionCount = 0;

unsigned long lastTransmit      = 0;

unsigned long lastSensorRead    = 0;

const unsigned long TRANSMIT_INTERVAL    = 5000;

const unsigned long SENSOR_READ_INTERVAL = 2000;

// ─── ESP-NOW receive callback ─────────────────────────────────────────────

void IRAM_ATTR onDataReceived(const uint8_t *mac, const uint8_t *data, int len) {

if (len < (int)sizeof(EspNowPacket)) return;

const EspNowPacket *pkt = (const EspNowPacket*)data;

if (pkt->type == PKT_IMAGE_CHUNK) {

if (pkt->chunkIndex == 0) {

imgBufLen = imgChunksRxd = imgLoRaNextChunk = 0;

imgTotalChunks = pkt->totalChunks;

imageReady = imgLoRaSending = false;

}

if (imgBufLen + pkt->dataLen < MAX_IMAGE_B64_SIZE) {

memcpy(imgBuf + imgBufLen, pkt->data, pkt->dataLen);

imgBufLen += pkt->dataLen;

imgChunksRxd++;

}

} else if (pkt->type == PKT_IMAGE_END) {

imgBuf[imgBufLen] = '\0';

imageReady = imgLoRaSending = true;

imgLoRaNextChunk = 0;

Serial.printf("[ESP-NOW] Image ready: %u bytes\n", imgBufLen);

}

}

// ─── Sensor reads ─────────────────────────────────────────────────────────

void readSensors() {

sensors.ph = PH_MIN + (random(0, 101) * (PH_MAX - PH_MIN) / 100.0f);

sensors.ph = round(sensors.ph * 100) / 100.0f;

int raw = analogRead(MOISTURE_PIN);

if (raw > 0 && raw < 4095)

sensors.moisture = constrain(map(raw, MOISTURE_DRY, MOISTURE_WET, 0, 100), 0, 100);

if (dsOK) {

ds18b20.requestTemperatures();

float t = ds18b20.getTempCByIndex(0);

if (t != DEVICE_DISCONNECTED_C) sensors.temp_ds18b20 = t;

}

if (bmpOK) {

float t = bmp.readTemperature(), p = bmp.readPressure();

if (!isnan(t)) sensors.temp_bmp = t;

if (!isnan(p)) sensors.pressure = p / 100.0f;

}

if (dhtOK) {

float t = dht.readTemperature(), h = dht.readHumidity();

if (!isnan(t)) sensors.temp_dht = t;

if (!isnan(h)) sensors.humidity = h;

}

}

// ─── LoRa: send sensor packet ─────────────────────────────────────────────

void transmitSensorData() {

transmissionCount++;

String pkt = "S:" + String(transmissionCount)

+ "," + String(sensors.ph, 2)

+ "," + String(sensors.moisture, 1)

+ "," + String(sensors.temp_ds18b20, 2)

+ "," + String(sensors.temp_bmp, 2)

+ "," + String(sensors.pressure, 2)

+ "," + String(sensors.temp_dht, 2)

+ "," + String(sensors.humidity, 2);

LoRa.beginPacket();

LoRa.print(pkt);

LoRa.endPacket();

Serial.printf("[TX#%d] %s\n", transmissionCount, pkt.c_str());

}

// ─── LoRa: relay next N image chunks ─────────────────────────────────────

void transmitImageChunks(int count) {

if (!imgLoRaSending || !imageReady) return;

uint16_t total = (imgBufLen + LORA_IMG_CHUNK_BYTES - 1) / LORA_IMG_CHUNK_BYTES;

for (int i = 0; i < count; i++) {

if (imgLoRaNextChunk >= total) {

imgLoRaSending = false;

Serial.println("[IMG] All chunks relayed via LoRa");

break;

}

size_t offset  = (size_t)imgLoRaNextChunk * LORA_IMG_CHUNK_BYTES;

size_t avail   = imgBufLen - offset;

size_t chunkSz = (avail > LORA_IMG_CHUNK_BYTES) ? LORA_IMG_CHUNK_BYTES : avail;

String header = "I:" + String(imgLoRaNextChunk) + ":" + String(total) + ":";

String chunk  = String(imgBuf + offset).substring(0, chunkSz);

LoRa.beginPacket();

LoRa.print(header + chunk);

LoRa.endPacket();

Serial.printf("[IMG] LoRa chunk %u/%u\n", imgLoRaNextChunk + 1, total);

imgLoRaNextChunk++;

}

}

// ─── Init ─────────────────────────────────────────────────────────────────

bool initLoRa() {

SPI.begin(LORA_SCK, LORA_MISO, LORA_MOSI, LORA_SS);

LoRa.setPins(LORA_SS, LORA_RST, LORA_DIO0);

if (!LoRa.begin(LORA_FREQUENCY)) return false;

LoRa.setSyncWord(LORA_SYNC_WORD);

LoRa.setSpreadingFactor(LORA_SPREADING_FACTOR);

LoRa.setSignalBandwidth(LORA_BANDWIDTH);

LoRa.setCodingRate4(LORA_CODING_RATE);

LoRa.setTxPower(LORA_TX_POWER);

return true;

}

void initEspNow() {

WiFi.mode(WIFI_STA);

WiFi.disconnect();

Serial.print(">>> Transmitter MAC (paste into CAM sketch): ");

Serial.println(WiFi.macAddress());

if (esp_now_init() == ESP_OK)

esp_now_register_recv_cb(onDataReceived);

else

Serial.println("ESP-NOW init failed");

}

void initSensors() {

dht.begin(); dhtOK = true;

ds18b20.begin(); dsOK = true;

Wire.begin(I2C_SDA, I2C_SCL);

if (bmp.begin()) bmpOK = true;

else Serial.println("BMP180 not found – using defaults");

pinMode(PH_PIN, INPUT);

pinMode(MOISTURE_PIN, INPUT);

randomSeed(analogRead(36));

}

// ─── Setup / Loop ─────────────────────────────────────────────────────────

void setup() {

Serial.begin(115200);

delay(2000);

Serial.println("\nLoRa Transmitter | ESP-NOW Relay");

initEspNow();

initSensors();

if (!initLoRa()) {

Serial.println("LoRa init failed – halting");

while (1) delay(1000);

}

readSensors();

Serial.println("System ready\n");

}

void loop() {

unsigned long now = millis();

if (now - lastSensorRead >= SENSOR_READ_INTERVAL) {

readSensors();

lastSensorRead = now;

}

if (now - lastTransmit >= TRANSMIT_INTERVAL) {

transmitSensorData();

transmitImageChunks(LORA_CHUNKS_PER_CYCLE);

lastTransmit = now;

}

delay(50);

}

/*

* SKETCH 1 – ESP32-CAM | Image Capture & ESP-NOW Sender

* Captures JPEG, base64-encodes it, sends chunks to Transmitter via ESP-NOW.

* ⚠ Flash Transmitter first, copy its MAC, paste into TRANSMITTER_MAC below.

*/

#include "esp_camera.h"

#include <WiFi.h>

#include <esp_now.h>

#include "mbedtls/base64.h"

// ─── Camera pins (AI-Thinker) ────────────────────────────────────────────

#define PWDN_GPIO_NUM   32

#define RESET_GPIO_NUM  -1

#define XCLK_GPIO_NUM    0

#define SIOD_GPIO_NUM   26

#define SIOC_GPIO_NUM   27

#define Y9_GPIO_NUM     35

#define Y8_GPIO_NUM     34

#define Y7_GPIO_NUM     39

#define Y6_GPIO_NUM     36

#define Y5_GPIO_NUM     21

#define Y4_GPIO_NUM     19

#define Y3_GPIO_NUM     18

#define Y2_GPIO_NUM      5

#define VSYNC_GPIO_NUM  25

#define HREF_GPIO_NUM   23

#define PCLK_GPIO_NUM   22

// ─── ⚠ Paste Transmitter MAC here (printed on its Serial at boot) ────────

uint8_t TRANSMITTER_MAC[] = {0xAA, 0xBB, 0xCC, 0xDD, 0xEE, 0xFF};

// ─── Timing ──────────────────────────────────────────────────────────────

// 86400000 = 24 h. Change to 60000 for 1-minute testing.

const unsigned long CAPTURE_INTERVAL = 86400000UL;

unsigned long lastCapture = 0;

// ─── ESP-NOW packet ──────────────────────────────────────────────────────

#define ESPNOW_CHUNK_BYTES 200

typedef enum : uint8_t {

PKT_IMAGE_CHUNK = 1,

PKT_IMAGE_END   = 2

} PktType;

struct __attribute__((packed)) EspNowPacket {

PktType  type;

uint16_t chunkIndex;

uint16_t totalChunks;

uint16_t dataLen;

char     data[ESPNOW_CHUNK_BYTES + 1];

};

volatile bool sendComplete = false;

volatile bool sendSuccess  = false;

void onDataSent(const uint8_t *mac, esp_now_send_status_t status) {

sendSuccess  = (status == ESP_NOW_SEND_SUCCESS);

sendComplete = true;

}

// ─── Camera init ─────────────────────────────────────────────────────────

void initCamera() {

camera_config_t cfg;

cfg.ledc_channel  = LEDC_CHANNEL_0;

cfg.ledc_timer    = LEDC_TIMER_0;

cfg.pin_d0 = Y2_GPIO_NUM; cfg.pin_d1 = Y3_GPIO_NUM;

cfg.pin_d2 = Y4_GPIO_NUM; cfg.pin_d3 = Y5_GPIO_NUM;

cfg.pin_d4 = Y6_GPIO_NUM; cfg.pin_d5 = Y7_GPIO_NUM;

cfg.pin_d6 = Y8_GPIO_NUM; cfg.pin_d7 = Y9_GPIO_NUM;

cfg.pin_xclk     = XCLK_GPIO_NUM;

cfg.pin_pclk     = PCLK_GPIO_NUM;

cfg.pin_vsync    = VSYNC_GPIO_NUM;

cfg.pin_href     = HREF_GPIO_NUM;

cfg.pin_sscb_sda = SIOD_GPIO_NUM;

cfg.pin_sscb_scl = SIOC_GPIO_NUM;

cfg.pin_pwdn     = PWDN_GPIO_NUM;

cfg.pin_reset    = RESET_GPIO_NUM;

cfg.xclk_freq_hz = 20000000;

cfg.pixel_format = PIXFORMAT_JPEG;

cfg.frame_size   = FRAMESIZE_QQVGA;  // 160x120 – LoRa friendly

cfg.jpeg_quality = 30;               // 0=best 63=worst

cfg.fb_count     = 1;

if (esp_camera_init(&cfg) != ESP_OK) {

Serial.println("Camera init failed – restarting");

delay(3000);

ESP.restart();

}

Serial.println("Camera ready");

}

// ─── ESP-NOW init ─────────────────────────────────────────────────────────

bool initEspNow() {

WiFi.mode(WIFI_STA);

WiFi.disconnect();

if (esp_now_init() != ESP_OK) return false;

esp_now_register_send_cb(onDataSent);

esp_now_peer_info_t peer = {};

memcpy(peer.peer_addr, TRANSMITTER_MAC, 6);

peer.channel = 0;

peer.encrypt = false;

return esp_now_add_peer(&peer) == ESP_OK;

}

// ─── Send one chunk with retry ────────────────────────────────────────────

bool sendChunk(const EspNowPacket &pkt, int maxRetries = 3) {

for (int attempt = 0; attempt < maxRetries; attempt++) {

sendComplete = sendSuccess = false;

esp_err_t err = esp_now_send(TRANSMITTER_MAC, (const uint8_t*)&pkt, sizeof(pkt));

if (err != ESP_OK) { delay(150); continue; }

unsigned long t = millis();

while (!sendComplete && millis() - t < 2000) delay(10);

if (sendSuccess) return true;

delay(200 * (attempt + 1));

}

return false;

}

// ─── Capture → base64 → ESP-NOW ──────────────────────────────────────────

void captureAndSend() {

Serial.println("[CAM] Capturing...");

camera_fb_t *fb = esp_camera_fb_get();

if (!fb) { Serial.println("Capture failed"); return; }

Serial.printf("Captured: %u bytes\n", fb->len);

size_t b64Len = ((fb->len + 2) / 3) * 4 + 4;

char  *b64Buf = (char*)malloc(b64Len);

if (!b64Buf) {

Serial.println("malloc failed");

esp_camera_fb_return(fb);

return;

}

size_t outLen = 0;

int rc = mbedtls_base64_encode(

(unsigned char*)b64Buf, b64Len, &outLen, fb->buf, fb->len);

esp_camera_fb_return(fb);

if (rc != 0) { Serial.printf("Base64 error: %d\n", rc); free(b64Buf); return; }

b64Buf[outLen] = '\0';

Serial.printf("Base64: %u bytes\n", (unsigned)outLen);

uint16_t totalChunks = (outLen + ESPNOW_CHUNK_BYTES - 1) / ESPNOW_CHUNK_BYTES;

Serial.printf("Sending %u chunks via ESP-NOW...\n", totalChunks);

int successCount = 0;

for (uint16_t i = 0; i < totalChunks; i++) {

EspNowPacket pkt;

pkt.type        = PKT_IMAGE_CHUNK;

pkt.chunkIndex  = i;

pkt.totalChunks = totalChunks;

size_t offset   = (size_t)i * ESPNOW_CHUNK_BYTES;

size_t avail    = outLen - offset;

pkt.dataLen     = (uint16_t)((avail > ESPNOW_CHUNK_BYTES) ? ESPNOW_CHUNK_BYTES : avail);

memcpy(pkt.data, b64Buf + offset, pkt.dataLen);

pkt.data[pkt.dataLen] = '\0';

if (sendChunk(pkt)) successCount++;

delay(60);

}

// End-of-image marker

EspNowPacket endPkt = {};

endPkt.type        = PKT_IMAGE_END;

endPkt.chunkIndex  = totalChunks;

endPkt.totalChunks = totalChunks;

sendChunk(endPkt);

free(b64Buf);

Serial.printf("Done: %d/%u chunks sent\n", successCount, totalChunks);

}

// ─── Setup / Loop ─────────────────────────────────────────────────────────

void setup() {

Serial.begin(115200);

delay(1000);

Serial.println("\nESP32-CAM | ESP-NOW Image Sender");

Serial.print("My MAC: "); Serial.println(WiFi.macAddress());

initCamera();

if (!initEspNow()) {

Serial.println("ESP-NOW init failed – halting");

while (1) delay(1000);

}

captureAndSend();

lastCapture = millis();

}

void loop() {

if (millis() - lastCapture >= CAPTURE_INTERVAL) {

captureAndSend();

lastCapture = millis();

}

delay(1000);

}